# LIDAR

Date: 06/11/25

Name: Mohammed Namir

Name: Naga Manohar

What is LIDAR?

LiDAR (Light Detection and Ranging) is a remote sensing method that measures distance by illuminating a target with laser light and measuring the time it takes for the reflection to return.

## 2. Working Principle

LiDAR emits rapid laser pulses and measures the Time of Flight (TOF) it takes for them to hit an object and reflect back. It calculates distance using the formula Distance=2Speed of Light×TOF​. By scanning in many directions, it builds a 3D map called a "point cloud."

## 3. Main Components

- Laser Emitter: Creates the light pulses.

- Scanner: Directs the laser pulses across the environment.

- Receiver (Photodetector): Captures the reflected light.

- GPS/IMU: Provide precise location and orientation of the sensor.

- Processing Unit: Calculates distances and assembles the 3D point cloud.

## 4. Types of LiDAR

- Terrestrial: Ground-based systems used in self-driving cars, robots, and surveying.

- Aerial: Mounted on aircraft (planes, drones) for topographic mapping.

- Spaceborne: Used on satellites or rovers to map planetary surfaces.

## 5. Advantages

- Produces highly accurate 3D maps (centimeter-level).

- Works effectively in both daylight and darkness.

- Captures precise geometry for obstacle detection.

## 6. Limitations

- Performance degrades in bad weather (fog, rain, or dust).

- Generally more expensive than cameras.

- Generates very large data files that require significant processing power.

### Why LiDAR is Useful for Rovers on Mars?

Mars rovers like Curiosity, Perseverance, or the upcoming ExoMars Rosalind Franklin are designed to operate autonomously in an extremely harsh and unknown environment.
 There are several reasons LiDAR is so helpful — let’s unpack them one by one.

#### a) Autonomous Navigation (No Real-Time Human Control)

- Why autonomy matters:
 Communication between Earth and Mars has a delay of 10–20 minutes (one-way).
 This means NASA can’t “drive” the rover in real time.
 → The rover must see and think for itself.

- How LiDAR helps:
 LiDAR gives the rover a real-time 3D view of its surroundings, allowing onboard computers to:

  - Detect rocks, slopes, or cliffs.

  - Choose paths that avoid obstacles.

  - Calculate safe routes on uneven terrain.

Essentially, LiDAR is the rover’s eyes for spatial awareness.

#### b) 3D Terrain Mapping & Surface Analysis

- LiDAR produces dense 3D point clouds, allowing the rover to model terrain with centimeter-level accuracy.

- This helps:

  - Identify geological formations (like rock layers or sand dunes).

  - Build 3D maps of craters, ridges, and cliffs for future exploration.

  - Plan manipulator arm movements precisely (for drilling or sampling).

Example:
 Before drilling a rock, a LiDAR scan ensures the drill head will contact the surface at the correct angle.

#### c) Obstacle Detection and Safe Traversal

- Mars is full of rocks, sand traps, and sharp slopes.

- A rover getting stuck (like Spirit in 2009) can end a mission permanently.

- LiDAR can scan ahead and:

  - Detect obstacles and drop-offs.

  - Measure the slope gradient.

  - Calculate traversable paths in real time.

This dramatically reduces the risk of accidents.

#### d) SLAM and Localization (Mapping Without GPS)

- There is no GPS on Mars — the rover must figure out its position using local sensors.

- LiDAR supports SLAM (Simultaneous Localization and Mapping):

  - It maps the environment while estimating the rover’s location within it.

  - Combined with IMUs and visual odometry, LiDAR makes this process more accurate and robust, especially when visual cues (like shadows or textures) are poor.

#### e) Low-Light and Dust Conditions

- Mars has frequent dust storms and low sunlight (especially at dawn/dusk).

- Cameras alone struggle in these conditions.

- LiDAR is active sensing — it emits its own light (laser), so it works in total darkness.
 → That’s a major advantage over passive vision systems.

3. LiDAR's Role and Integration to ROS

LiDAR is one of the very fundamental sensors in robotics, and ROS is built in such a way that its use is easy. It is the integration of LiDAR that makes a robot perceive its surroundings for mapping and navigation.

This link is established through a few key ROS concepts:

ROS 2 Driver: The "Translator" - The connection starts with a particular ROS 2 driver package for the LiDAR hardware. This driver is a ROS 2 Node-a program-whose job is to "translate" the raw, proprietary data from the sensor into a standardized format understandable by all the other components of ROS 2.

ROS 2 Topics (The "Data Channel"): Once the driver node launches, it will begin to publish this standardized data onto a ROS 2 Topic. A topic is like a public data channel. For LiDAR, this topic is often named /scan (for 2D LiDAR) or /points (for 3D LiDAR).

Standardized messages use some form of standardized message format on the data in communication for the subject at hand.

sensor_msgs/LaserScan : This is the message that is used for 2D LiDAR. It is a simple, efficient array of distance measurements.

sensor_msgs/PointCloud2: This is the message type utilized for 3D LiDAR, comprising of a large set of (x, y, z) points that, together, form a 3D "point cloud".

This is the standardized system that makes LiDAR so useful in ROS. Any ROS 2 node can subscribe to the LiDAR topic to get this clean, ready-to-use data. This allows powerful, separate nodes to perform complex tasks:

Obstacle Detection: A "safety" node can subscribe to the /scan topic, and if any distance measurement is too short, it overrides the robot's motion to command a stop and thus avoid a collision. Mapping: A SLAM node such as slam_toolbox can subscribe to the /scan topic to use the laser scans to build a 2D map of an environment. Navigation: With the map created, the ROS 2 Navigation Stack (Nav2) subscribes to the /scan topic to know where, on the map, a robot currently is (localization) and to detect new, previously unmapped obstacles such as a person passing by so it can plan a safe path around them.